Transfection

Introduction

Creating retrovirus for a plasmid.

Materials

› Material

›

Procedure

Procedure

Day 1: Plating Cells

1. Use medium without PenStrep (improves transfection efficiency)

2. Plate cells as follows:

10 cm dish:

2.5 × 106 cells

10 mL medium

6-well plate:

0.5 × 106 cells per well

2 mL medium per well

Day 2: Transfection

3. Remove media, replace with fresh media from Plate E cells

4. Prepare transfection mix (per well):

Tube 1:

5 μg ( = 50 μL) pCL-Eco plasmid

550 μL OptiMEM

Aliquot 600 μL into 15 mL Falcon tubes Add 10 μg of transfer plasmid

Tube 2:

45 μL Mirus transfection reagent

655 μL OptiMEM

Combine tubes:

Page 1 of 2

Add 700 μL of Tube 2 mix into each Falcon tube

Incubate at room temperature for 20 min

5. Add dropwise to cells

Day 3

6. Remove old media

7. Add fresh TCM media

Day 4

8. Harvest first round of virus (supernatant)

9. Store in fridge

10. Add fresh TCM media to cells

Day 5

11. Harvest remaining virus, combine with Day 4 supernatant

12. Spin at 1600 rpm for 4 min

13. Aliquot:

2 mL per 5 mL Eppi or use directly

14. Freeze at -80°C

15. If using fresh virus, filter through 0.45 μm filter

Page 2 of 2