Tissues except SI

- Fix your samples with PFA 2-4%. RT then 4’C overnight (use capsule PFA for freshness)

  - Kidney- cut 1 kidney in half, fix 24h

  - Liver- separate into lobes, after a couple hours of fixation, cut into smaller pieces for overnight fixation, fix 48h

  - Spleen- put into fixation 40sec-1min post death

- Transfer into 15% sucrose in 1×PBS at 4°C with gentle shaking until tissue sinks

- Transfer into 30% sucrose in 1×PBS at 4°C with gentle shaking until tissue sinks (overnight)

- Transfer into 50/50 with 30% sucrose/OCT for 1-4h at RT with gentle shaking

- Remove excess liquid surround the tissue by absorption with Kim-wipe prior to freezing. Otherwise, the liquid will form ice crystal on the surface of tissue and prevent tissue attachment to OCT and cause a lot of difficulties during sectioning

- Transfer into mold with OCT for 1-4h with gentle shaking

- Label the cryomold to mark the orientation of the tissue.

- Using forceps, lower the cryomold containing embedded tissue into the isopentane without fully submerging. Perform in chem hood to avoid fumes.

  - If using shallow mold, put on dry ice to freeze

- Once frozen, place the cryomold on a piece of aluminum foil on dry ice for 30 min and let isopentane fumes evaporate

For Small Intestine (fix first as it degrades quick)

- Flush small intestine with BD Fix/Perm solution. Remove as much of the mesentery to keep intestine straight. Holes in SI will make it difficult to flush. (BD fix/perm diluted with 1×PBS at a ratio of 1:2)

- Insert crochet needle into proximal part of small intestine. Wet needle prior to insertion.

- Place needle with SI into box and cover with 2% PFA

- Remove SI from crochet needle and cut longitudinally except the proximal end (for placement later).

  - If SI starts to bunch up, STOP.

  - Cut intestine longitudinally with scalpel until it can be removed easily from needle.

- Dip into PBS several times to clean out remaining feces

- Swiss roll intestine. Secure with 30G needle.

- Fix with 4% PFA overnight at 4’C in 6 well plate

- Rinse with PBS for 10 minutes 3X

- Transfer into 15% sucrose in 1×PBS at 4°C until tissue sinks

- Transfer into 30% sucrose in 1×PBS at 4°C until tissue sinks/overnight.

- Transfer into 50/50 with 30%sucrose/OCT with GENTLE shaking

- Transfer into OCT for 1-4hours

- Label the cryomold to mark the orientation of the tissue.

- Using forceps, lower the cryomold containing embedded tissue into the isopentane without fully submerging. Perform in chem hood to avoid fumes.

  - If using shallow mold, put on dry ice to freeze

- Once frozen, place the cryomold on a piece of aluminum foil on dry ice for 30 min and let isopentane fumes evaporate

For colon/cecum (roll first post fix b/c it will get stiff)

- Flush colon with BD Fix/Perm solution. Remove as much of the mesentery to keep intestine straight. Holes in SI will make it difficult to flush. (BD fix/perm diluted with 1×PBS at a ratio of 1:2)

- Remove hard feces by squeezing out of colon.

- Insert crochet needle. Wet needle prior to insertion.

- Place needle with SI into box and cover with 4% PFA

- Cut open longitudinally and wash with PBS

- Swiss roll ASAP.

  - **Colon is a thicker tissue and becomes stiff fast, making it hard to roll post fix

  - 30G needle will not puncture colon, use 27G

- Fix overnight at 4’C in 6 well plate

- Rinse with PBS for 10 minutes 3X

- Transfer into 15% sucrose in 1×PBS at 4°C until tissue sinks

- Transfer into 30% sucrose in 1×PBS at 4°C until tissue sinks/overnight. T

- Transfer into 50/50 with 30%sucrose/OCT with GENTLE shaking for 1-4h

- Transfer into OCT for 1-4hours

- Label the cryomold to mark the orientation of the tissue.

- Using forceps, lower the cryomold containing embedded tissue into the isopentane without fully submerging. Perform in chem hood to avoid fumes.

  - If using shallow mold, put on dry ice to freeze

- Once frozen, place the cryomold on a piece of aluminum foil on dry ice for 30 min and let isopentane fumes evaporate