Hematoxylin and Eosin Staining on paraffin sections

(adapted from Xenium 10X protocol)

Deparaffinization

Deparaffinization steps should be performed in a fume hood due to the hazardous nature of xylene.

- Section FFPE tissues at 5um.

- Let dry overnight at RT.

- Place slides in a section dryer oven and incubate uncovered at 60°C for 30 min-1h

- Remove from the oven and cool down to room temperature for 7 min. Move the slides to each successive jar.

- Xylene Jar 1 and incubate for 10 min.

- Xylene Jar 2 and incubate for 10 min.

- 100% Ethanol Jar 1 for 3 min.

- 100% Ethanol Jar 2 for 3 min.

- 96% Ethanol Jar 1 for 3 min.

- 96% Ethanol Jar 2 for 3 min.

- 70% Ethanol Jar for 3 min.

- Nuclease-free water Jar for 20 sec.

H & E Stain

- Milli-Q Water Jar 1 for 2 min

- Hematoxylin Solution Jar for 20 min at room temperature.

- Milli-Q Water Jar 2 for 1 min at room temperature.

- Milli-Q Water Jar 3 for 1 min at room temperature

- Milli-Q Water Jar 4 for 1 min at room temperature. (gently shake off as much water as possible prior to bluing)

- Bluing Solution Jar for 1 min at room temperature.

- Milli-Q Water Jar 5 for 1 min at room temperature.

- 70% Ethanol Jar for 3 min at room temperature.

- 95% Ethanol Jar 1 for 3 min at room temperature.

- Eosin Solution Jar for 10-20 sec at room temperature.  (Adjust timing based on how dark it seems)

- 95% Ethanol Jar 2 for 30 sec at room temperature.

- 95% Ethanol Jar 3 for 30 sec at room temperature.

- 100% Ethanol Jar 1 for 30 sec at room temperature.

- 100% Ethanol Jar 2 for 30 sec at room temperature.

- Xylene Jar 1 for 3 min at room temperature.

- Xylene Jar 2 for 3 min at room temperature.

- Apply cytoseal mounting media and coverslip