Protocol for Thawing Frozen Cells

(modified from https://www.thermofisher.com/it/en/home/references/gibco-cell-culture-basics/cell-culture-protocols/thawing-cells.html)

The thawing procedure is stressful to frozen cells, and using good technique and working quickly ensures that a high proportion of the cells survive the procedure.  As with other cell culture procedures, we recommend that you closely follow the instructions provided with your cells and other reagents for best results.

NOTE:

- Thaw frozen cells rapidly (< 1 minute) in a 37°C water bath.

- Dilute the thawed cells slowly, using pre-warmed growth medium.

- Plate thawed cells at high density to optimize recovery.

- Always use proper aseptic technique and work in a laminar flow hood.

- Always wear personal protective equipment, including a face mask or goggles.  Cryovials stored in liquid-phase present a risk of explosion when thawed.

- Some freezing media contain DMSO, which is known to facilitate the entry of organic molecules into tissues. Handle reagents containing DMSO using equipment

Materials

- Cryovial containing frozen cells

- Complete growth medium, pre-warmed to 37°C

- Disposable, sterile centrifuge tubes

- Water bath at 37°C

- 70% ethanol

Protocol

The following protocol describes a general procedure for thawing cryopreserved cells.  For detailed protocols, always refer to the cell-specific product insert.

- Remove the cryovial containing the frozen cells from liquid nitrogen storage and immediately place it into a 37°C water bath.

- Quickly thaw the cells (< 1 minute) by gently swirling the vial in the 37°C water bath until there is just a small bit of ice left in the vial.

- Transfer the vial into a laminar flow hood.  Before opening, wipe the outside of the vial with 70% ethanol.

- Add 1mL pre-warmed complete growth medium dropwise into the cryovial containing frozen cells.

- Gently resuspend the cells in complete growth medium, and transfer them into a Conical Centrifuge Tube (Falcon™ 15mL or 50mL Conical Centrifuge Tubes) with the desired amount of pre-warmed complete growth medium appropriate for the number of your cells (e.g. 10mL for up to 107 cells, 20mL for up to 5x107 cells).

- Centrifuge the cell suspension at approximately 400 × g for 6 minutes.  The actual centrifugation speed and duration varies depending on the cell type.

- After the centrifugation, check the clarity of supernatant and visibility of a complete pellet. Aseptically decant the supernatant without disturbing the cell pellet.

- Gently resuspend the cells with complete growth medium or with the buffer of interest.

ADDITIONAL NOTE

- If you are going to perform viability staining with Live Dead from Invitrogen or with Zombie Fixable Viability Kit from BioLegend, resuspend cells with PBS without calcium and magnesium in order to remove residual of Fetal Bovine Serum (FBS) or Fetal Calf Serum (FCS) contained in the complete growth medium used for thawing procedure.

- If you have high content of dead cells, you can use DNase I for removing dead cells’ DNA during sample preparation. We add DNase both to the complete growth medium used for thawing cells, both to the PBS used for washing cells after thawing. We use “Deoxyribonuclease I from bovine pancreas” from SIGMA ALDRICH DN25 (stock 5mg/mL, final concentration: 20 ug/mL).