Cas9 – crRNA (RNP) electroporation using the Neon System

MRC 11/2019 v1.0

Materials

- Neon Electroporator

- Electroporation pipette

- 100 ul kit (tips + R buffer + E buffer)

- Recombinant Cas 9 (6.5mg/ml; QB3 Berkley Lab, 39 pmol/uL)

- Synthetic RNA (Synthego, ready to use)

- Cell of interest

Before

Check if conditions have been previously optimized: https://www.thermofisher.com/us/en/home/life-science/cell-culture/transfection/neon-transfection-system/neon-transfection-system-cell-line-data.html

If not yet described – Use Optimization protocol and use GFP/Ametrine plasmid in your cells of interest. It highly depends of cell type. Try the 24 protocols and assess for efficiency and survival.

Amount of Reagents

For each electroporation sample, the amount of plasmid DNA/siRNA, cell number, and volume of plating medium per well are listed in the table below. Be sure to use Resuspension Buffer T for primary suspension blood cells and Resuspension Buffer R of all other cell types.

| Format | Cell Type | DNA (μg) | siRNA (nM) | NeonTip | Vol. plating medium | Cell no. | Resuspension Buffer R or T |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 96-well | Adherent | 0.25–0.5 | 10–200 | 10 μL | 100 μL | 1–2 × 104 | 10 μL |
|  | Suspension | 0.5–1 |  |  |  | 2–5 × 104 | 10 μL |
| 48-well | Adherent | 0.25–1 | 10–200 | 10 μL | 250 μL | 2.5–5 × 104 | 10 μL |
|  | Suspension | 0.5–2 |  |  |  | 5–12.5 × 104 | 10 μL |
| 24-well | Adherent | 0.5–2 | 10–200 | 10 μL | 500 μL | 0.5–1 × 105 | 10 μL |
|  | Suspension | 0.5–3 |  |  |  | 1–2.5 × 105 | 10 μL |
| 12-well | Adherent | 0.5–3 | 10–200 | 10 μL | 1 mL | 1–2 × 105 | 10 μL |
|  | Suspension | 0.5–3 |  |  |  | 2–5 × 105 | 10 μL |
| 6-well | Adherent | 0.5–3 (10 μL) 5–30 (100 μL) | 10–200 | 10 μL/100 μL | 2 mL | 2–4 × 105 | 10 μL/100 μL |
|  | Suspension | 0.5–3 (10 μL) 5–30 (100 μL) |  |  |  | 0.4–1 × 106 | 10 μL/100 μL |
| 60 mm | Adherent | 5–30 | 10–200 | 100 μL | 5 mL | 0.5–1 × 106 | 100 μL |
|  | Suspension | 5–30 |  |  |  | 1–2.5 × 106 | 100 μL |
| 10 cm | Adherent | 5–30 | 10–200 | 100 μL | 10 mL | 1–2 × 106 | 100 μL |
|  | Suspension | 5–30 |  |  |  | 2–5 × 106 | 100 μL |

Preparing Cells and RNP mix

- Cultivate the required number of cells (see table below for cell amount) such that the cells are 70–90% confluent on the day of the experiment.

- Rehydrate the sgRNA stock like this: 3 nmol of sgRNA powder plus 30 ul TE buffer. This will give you a 100 uM (100 pmol/ul) sgRNA stock solution (store at -80C).

- Dilute sgRNA like this: 6 ul of 100 uM sgRNA stock plus 14 ul nuclease-free water.

- Prepare plates by filling the wells with the appropriate volume of culture medium containing serum and supplements without antibiotics as described in the table below and pre-incubate plates at 37°C in a humidified 5% CO2 incubator.

- Prepare this mix:

- 1.3 ul of diluted sgRNA guide (30 uM)

- 1 ul TrueCut Cas9 6 uM or 1 ul

- 12.4 ul Buffer R Incubate this mix for 20 minutes at RT

- During those 20 min, prepare a suspension with your cells (50,000-150000 cells, depending on your cells (10 to 20 million cells/ml) in an Eppendorf, and prepare electroporator.

- Wash cells once with PBS1x and once with Buffer R.

- Resuspend the pellet in 10/100 ul of the CRISPR/Cas9 MIX

- Proceed to Electroporation

Electroporation

- Fill the NeonTube with 3 mL Electrolytic Buffer (use Buffer E for 10 μL NeonTip and Buffer E2 for 100 μL NeonTip).

- Insert the NeonTube into the NeonPipette Station until you hear a click sound (Figure 1).

- Transfer the appropriate amount of plasmid DNA/siRNA into a sterile, 1.5 mL microcentrifuge tube.

- Add cells to the tube containing plasmid DNA/siRNA and gently mix. See the table on back side for cell number, DNA/siRNA amount, and plating volumes to use.

- Press the push-button on the NeonPipette to the second stop to open the clamp and insert the top-head of the NeonPipette into the NeonTip until the clamp fully picks up the mounting stem of the piston. Gently release the push-button, continuing to apply a downward pressure on the pipette, ensuring that the tip is sealed onto the pipette without any gaps (Figure 2A and B).

- Press the push-button on the NeonPipette to the first stop and immerse the NeonTip into the cell-DNA/siRNA mixture. Slowly release the push-button on the pipette to aspirate the cell-DNA/siRNA mixture into the NeonTip.

Note: Avoid air bubbles during pipetting as air bubbles cause arcing during electroporation. If you notice air bubbles in the tip, discard the sample and carefully aspirate fresh sample into the tip again without any air bubbles.

- Insert the NeonPipette with the sample vertically into the NeonTube placed in the NeonPipette Station until you hear a click sound.

- Ensure that you have selected the appropriate electroporation protocol and press Start on the touchscreen (Figure 5).

- The Neondevice automatically checks for the proper insertion of the NeonTube and NeonPipette before delivering the electric pulse.

- The touchscreen displays Complete to indicate that the electroporation is complete.

- Remove the NeonPipette from the NeonPipette Station and immediately transfer the samples from the NeonTip by pressing the push-button on the pipette to the first stop into the prepared culture plate containing prewarm medium with serum and supplements but without antibiotics (Figure 6).

Discard the NeonTip into an appropriate biological hazardous waste container.

- Repeat Steps 5–11 for the remaining samples.

Change the NeonTips after using it twice and NeonTubes after 10 usages. Use a new NeonTip and NeonTube for each new plasmid DNA/siRNA or cell type.

- Gently rock the plate to assure even distribution of the cells. Incubate the plate at 37°C in a humidified CO2 incubator.

- If you are not using the Neondevice, turn the power switch on the rear to OFF.

- Assay samples to determine the transfection efficiency (e.g., fluorescence microscopy or functional assay) or gene knockdown (for siRNA).

- Based on your initial results, you may need to optimize the electroporation parameters for your cell type. Refer to the manual for using the 18-well or preprogrammed, 24-well optimization protocol on the Neondevice.

Checking the cleavage efficiency

- Resuspend the pellet in 20 ul of 1x ThermoPol (New England BioLabs B9004S) in an eppendorf. Lyse at 95C for 10 min.

- Cool the lysate at 4C (in ice) during 5 min.

- Add 2.5 ul of Proteinase K 20 mg/ml and incubate at 55 C for 1 hr.

- Inactivate proteinase K incubating 15 min at 95C.

- Cool the lysate at 4C (in ice) for at least 5 min.

- Use the AmpliTaq Gold 360 master mix kit from Life Technologies (REF 43988881, -20C, CRISPR box) to set up the PCR with the designed primers for each CRISPR.

Mix

AmpliGold 2x 25 ul

sgDNA 2 ul

Primers (10 uM F/R) 1 ul

DDW 22 ul

Program:

1. 95C 10 min

2. 95C 30 sec

3. 56C 30 sec

4. 72C 60 sec/per kb (30 sec for an amplicon of 500 bp)

Go 4 to 2, 40 cycles

5. 72C, 7 min

6. 4C, forever

- Run 5 ul in a gel. You should see one single band at around 500 bp.

- Use the GeneArt Genomic Cleavage Detection kit (Life Technologies Cat A24372) to digest your PCR product:

- Combine in 2 different 8-well strip tubes (one will – and the other will be +)

Mix

PCR product 3 ul

10x Detection Reaction Buffer

1 ul DDW 5 ul

Program:

1. 95C 5 min

2. 95C-85C -2C/ sec

3. 85C-25C -0.1C/sec

4. 4C, forever

- Following the completion of the re-annealing, proceed to enzyme digestion or store the reaction at -20C. Add 1 ul of Detection Enzyme to the test sample (+), mix well. Add 1 ul of DDW to the test sample (-), mix well.

- Incubate at 37C for 1 hr.

- Vortex briefly and spin down.

- Place at 4C.

- Immediately load the entire 10 ul on a 2% gel.