# Setting up PCR

- Use computer program to calculate correct volumes for PCR mixture based on the number of DNA samples that need to be run.  (#of samples + 2)

- Make sure that all reagents are diluted to the concentration used by the program.  Also check that you have the correct oligos.

- Add 1l of each DNA sample to separate wells on a PCR plate, check wells.

- Combine all reagents other than the Taq and the DNA itself.

- Remove the Taq from the freezer and add to the mixture, vortex.

- Pipette 24 l of the mixture into each well containing DNA.

- Start desired program on PCR machine.  Once the block reaches 95 C, place the plate into the machine and seal with film immediately.

- Close lid of machine until program is complete.