Staining Blood:

- Spin blood tubes at 5000 RPM for 5 minutes

- Suck off PBS (clear supernatant)

- Resuspend blood pellet in 1x Ack; Incubate ~10 minutes at room temperature

- Spin at 5000 RPM for 5 minutes (should see a white pellet)

- Suck off supernatant leaving a white pellet behind

- Resuspend pellet in 150 microliters of FACs buffer and trasfer to a “v”-bottom plate

- Spin the plate at 2000 RPM for 1 minute; flick the plate to get rid of supernatant

- Add 50 microliters of stain to each well and mix with pipette

- Incubate the plate for about 20 minutes on ice

- Add 150 microliters of FACs buffer to the well; spin at 2000 RPM for 1 minute and flick the plate

- Resuspend pellet in 150 microliters of FACs buffer; spin 2000 RPM for 1 minute and flick the plate

- Resuspend the pellet in 150 microliters FACs buffer and transfer to the FACs tubes

- Analyze the samples on the FACs machine