Basic Immunofluorescent Staining Protocol for OCT sections

- Section frozen sections at 10um.

- Dry at RT for 1 hour

- Pap pen around tissue

- Wash 3x PBS for 10min

- Permeabilize if needed (nuclear or intracellular markers) with 0.1% Triton-X in PBS at RT in humidified chamber.

  - If surface markers only, skip to step 6.

  - All incubation steps after this will be in humidified chamber.

- Block with DAKO block 1h

- Wash 3x PBS for 10min

- Primary Ab overnight at 4’C in humified chamber. Dilute antibody with DAKO antibody dilution buffer.

Next Day

- Wash 3x PBS for 10min

- If secondary is needed: Dilute secondary antibody with DAKO antibody dilution buffer (1:500-1:1000). Incubate at RT for 1-2h.

- Wash 2x PBS for 10min.

- DAPI (dilute 10ul in 1ml). Incubate 1min

- Dump DAPI off

- Dry slide as much as possible.

- Add Vectashield vibrance mounting media and cover with glass coverslip

- Let cure for at least 1-2h in the dark at RT (best O/N)

Note: 0.1% Tween 20 in PBS can be substituted for PBS during wash steps