## Western blot buffers

## Western Lysis Buffer

### IPN150

- 50mM Tris/HCl pH 7.6

- 150mM NaCl

- 5mM MgCl2

- 0.1% NP40

add fresh:

- 1x protease inhibitor (Sigma)

- 1mM DTT

- Benzamidine (1mM)

If analyzing phosphorylation:

- 1:100 Phosphatase Inhibitor Cocktail 2 (Sigma)

- 1:100 Phosphatase Inhibitor Cocktail 3 (Sigma)

## 10x Transfer buffer

For 1000ml

- 30g Tris base

- 144g Glycin

- H20

## 1x Transfer buffer

- 100ml 10x transfer buffer

- 700ml H20

- 200ml Methanol

## 10x TBS

- 24.3g Tris base

- 88g Nacl

- 1000ml H20

- adjust pH to 7.6

## 1x TBS-T

- 100ml 10x TBS

- 900ml H20

- 1ml Tween 20

# Protein Lysis

- Prepare lysis buffer (see Buffers)

- Pellet cells and resuspend in Lysis Buffer

- Keep on ice for 30 minutes

- Spin at 16000 g for 10 min at 4°C

- Transfer supernatant into new tube

- freeze at -80°C

# Protein quantification with BCA assay

## Prepare protein standard

- Use a 96-well flat bottom plate

- Add 184 ul H20 to the first well of the first two columns.

- Add 100 ul H20 to the remaining wells of the first two columns

- Add 16 ul BSA (2 mg/ml) to the first well of each column and then do a 1:2 dilution by transferring 100ul down the column,

- Standard curve: 16ug, 8ug, 4ug, 2ug, 1ug, 0.5ug, 0.25ug, and 0ug

## Prepare samples

- If expecting high protein concentration: Add 2 ul sample to 98ul H20.

- If expecting lower protein concentration: Add 5 ul sample to 95ul H20.

## BCA Assay

- Mix reagent A to B in 50:1 mixture.

- Add 100ul to each well.

- Incubate at 37°C for 10-15 min

- Read plate at 560nm

# Run Western Blot

## Prepare sample

- total volume depends on the gel usually between 15 and 20ul can be loaded

- for 20ul

  - 5ul 4x loading dye

  - 2ul 10x reducing agent

  - 13ul sample

    - keep ug protein between the samples consistent, fill with lysis buffer

- heat at 70°C for 10min

- spin down shortly

- keep on ice

## Load and Run gel

- Gel used: Bis-Tris NuPage 4-12%

- prepare MES running buffer by diluting the 20x stock

- fill chamber with running buffer

- load 8ul PageRuler™ Plus Prestained Protein Ladder

- load samples

- add 500ul Antioxidant to the inner chamber

- run at 120V until desired location

## Transfer

- soak membrane in methanol for 30s

- soak membrane in transfer buffer > 2min

- prepare transfer cassette (see image

- make sure all bubbles are removed between gel and membrane

- fill with transfer buffer

- add ice pack into chamber

- run at 70V for 90min at 4°C

## Blocking

- Block membrane in TBS-T + 5% BSA (or milk powder) for 1h at room temperature

- wash 3x with TBS-T for 5 min each.

## Staining

- stain with primary Ab in TBS-T + 5% BSA over night at 4°C

- wash three times

- add secondary antibody TBS-T + 5% BSA, incubate for 1 hour at room temperature

- wash at least three times

## Developing

I used Amersham™ ECL™ Prime Western Blotting Detection Reagent

- Mix solutions A and B at 1:1, if primary ab is strong, can add PBS to dilute

- Drain excess wash buffer from the washed membrane and place it protein side up in a suitable box or on a sheet of plastic sheet protecter or other suitable clean surface. Add detection reagent onto the membrane and make sure it completely covers the membrane

- Incubate for 45sec at room temperature

- Drain off excess detection reagent by holding the membrane edge gently against a tissue