Purification of Antibodies:

Equipment:

HiTrap Protein G column –

Peristaltic Pump or syringe

Collection tubes (FACS tubes work really well) with 100 ul of Tris pH 9.0

Protocol:

Important things to remember:

Peristaltic Pump:

- Run Sodium Phosphate buffer pH 7.0 into the tubing to equilibrate it until it begins dripping out the side with the metal lock.  Stop the flow.

- Place column onto a metal stand

- Attach the metal lock to the column using the red adaptor then continue the flow.  This is known as adding it ‘drop-to-drop’, which prevents air from entering the column.

- Run 5x the volume of the column (25 ml) of Sodium Phosphate buffer pH 7.0  over the column and save the flow through.

- Begin running your antibody supernatant over the column…make sure to use a new collection tube once the supernatant reaches the column.

- Once all of your supernatant has been collected, run another 5 volumes of Sodium Phosphate buffer pH 7.0.

- Elute the antibody using 0.1M Glycine buffer, pH 2.8.  20 fractions of approximately 1 ml each is good.  For this small a volume, 30 ml syringes work well to add the glycine.  Just make sure to put the syringe on ‘drop-to-drop’ to avoid air entering the column.

- Once eluted, attach the column onto the metal stand once again, and run 5 bed volumes of 20% EtOH through it.  The column can be stored at 4C until later use.  The tubing can stay attached until later use.

- Set up a 96-well plate with the following for each fraction (don’t forget pos and neg controls too!):

  - 20 ul sample

  - 20 ul biorad protein assay dye (Cat # 500-0006)

  - 60 ul H20

- Collect the fractions that turn the darkest blue – these contain the eluted antibody!

- Use spin collection tubes to concentrate the antibody (see tube directions for how)

- Use a protein assay kit (BioRad) to analyze the concentration of your antibody.