Edward Yang – 11/1/08

16% PFA – for 500 mL

Materials

350mL DI H20

80 grams of dry PFA

50 mL 10x PBS

Very large beaker (1 liter flat bottom)

500mL beaker (round)

Magnetic stir/ heat plate

Magnetic stir bar

Thermometer

pH meter

10-12N HCL

10-12N NaOH

Method – EVERYTHING is done in the chemical hood

- Fill 500mL beaker with 350mL DI H20 and place into very large beaker. Fill large beaker with H20 until covering ~2/3 of 500mL beaker. Place thermometer in outer water bath.

  - Heat to between 60-70 degrees C. Do not exceed 70 degrees C as that is the flash point of PFA.

- Weigh out 80g of dry PFA in the hood. Place magnetic stir bar (and start stirring) into 500mL beaker and then pour in dry PFA slowly.

- Use plastic pipet to pipet in 1-2 pipet-full’s worth of NaOH. Watch temperature to make sure it’s at ~65 degrees C.

  - As PFA begins to go into solution, pipet in NaOH dropwise to improve solubility, conservatively.

- Add NaOH dropwise until PFA completely goes into solution. The color of the solution should be completely clear/transparent.

  - Once in solution, pour in 50mL of 10x PBS, fill the rest up to the 500mL line with DI H20, and stir until mixed.

- Turn off heat, pour out outer water bath, and fill with cold/RT water. Cool PFA solution until room temperature.

- pH the solution to about 7.2-7.4 pH using HCL.

- Sterile filter PFA using ~0.1 micron filter. Label with date and 16%.