Procedure:

- End-Repair

- Use End-it DNA End-repair Kit from Epicentre

uL

DNA+H2O                34

10X End Repair Buffer                 5

2.5mM dNTPs Mix                 5

10mM ATP Mix                 5

END-IT enzyme mix                 1

total                  50

- Incubate for 45 min at room temperature.

- Mini-elute purification (Zymo DNA Clean&Concentrator). Elute it twice in 16ul of EB

- Total elute volume is 32uL.

2. Addition of an ‘A’ Base to the 3’ End of the DNA fragments

uL

DNA from above		32

10x NEB buffer 2		5

1mM dATP			10

5 U/mL Klenow (exo-)		3

uL

DNA from above		32

10x NEB buffer 2		5

1mM dATP			10

5 U/mL Klenow (exo-)		3

- Incubate for 30 min at 37C.

- Mini-elute purification (Zymo DNA Clean&Concentrator).

- Elute it in 8ul of EB

3. Ligation of Adapters to the Ends of the DNA Fragments

uL

DNA               8

2X Ligation Buffer              10

Adaptor               1  ( Adaptor was diluted 1:10 in DDW before add it)

Quick Ligase              1

total              20

- Incubate for 20 min. at room temperature. Add 1 uL of NEB USER. Incubate at 37C for 15 minutes.

- Mini-elute purification (Zymo DNA Clean&Concentrator). Elute twice in 10ul of EB (reuse). Total elute volume is 10uL.

Note: If you use NEB adaptor, remember to add 1uL USER Enzyme to incubate at 37C for 15 min after ligation.

4. Size selection of library using Gel purification

Materials

Nanosep MF Filter tube(VWR Cat.29300-642)

SYBR Gold (Invitrigen S11494)

Method

Prepare the Gel

• Prepare 8 % polyacrymide gel or buy:

40 % acryl (29:1)             6mL

5X TBE                            3mL

ddH2O                             21mL

10%APS                          300uL(Fresh Made)

TEMED                           40uL (Add it in fumehood)

Running buffer is 0.5X TBE . (600 ml/gel)

Run the Gel

• Add 6x Bromo phenol blue/xylene cyanol loading dye into the amplified library to make it 1X.

• Choose log ladder, load 10uL into the first line and the last line,.

• Load all samples into 8% polyacrymide gel every other line.

• Run at 200volt for 70 min, when the yellow dye reaches the bottom of the gel.

Stain and Cut the gel

• Gel Staining

Use a glass container for staining

Staining Buffer( Prepare in Glassware bottle)

(100ml 1X TAE, 10uL 10,000 x SYBR Gold )

Put gel into staining buffer and gently shake for 10min.

Cover the plate by foil to avoid the light.

• Excise bands from 200bps to 500bps with a clean scalpel.

• Minced the gel piece by using a 0.5ml tube with holes in the bottom. (Put the 0.5mL tube into a 1.5mL or 2mL tube, and centrifuge at 14k for 3 min. Make holes with a needle.)

• Add 400uL EB Buffer to cover the gel pieces, Shake for O/N at 37oC.

DNA Precipitation

• Transfer the supernatant to Nanoseq column , spin 2 min @ 14k.

• Transfer the rest of things in the sample tube to the column, spin 2 min @ 14k.

• Add EB Buffer to bring to a total volume of 400ul.

• Add 45uL  3M NaOAC(ph 5.2) and vortex to mix.

• Add 2ul glycogen(20mg/ml) and 1.2mL cold 100% ETOH, vortex.

• Freeze at -80°C for 1h (or O/N), spin 13.8K for 30min in cold room or @4oC. Remove supernatant.

• Wash with 1mL 70% EtOH, vortex, spin for 15min @13.8k rpm, remove all traces of EtOH, air dry

5 minutes (pellet turns clear).

• Resuspend precipitated DNA in 10ul EB.

5. PCR Amplification of Adaptor-ligated DNA

Test run:

uL

Adaptor ligated CHIP DNA                   1 + 0.5 of NFW

NEBNext High Fidelity 2X PCR Master Mix                   7.5

Index Primer (dilute 1:10)                   3

Universal PCR Primer (dilute 1:10                   3

total                    15

PCR Cycling Conditions:

| Temp | Time | Cycle |
| --- | --- | --- |
| 95 | 10 min | 1 |
| 95 | 30 sec | 30 cycles |
| 65 | 1 min |  |
| 72 | 1 min |  |

Calculate the number of cycles to use for amplification:

Cycle Number for start of plateau

Amplification:

uL

Adaptor ligated CHIP DNA                   10

NEBNext High Fidelity 2X PCR Master Mix                   25

Index Primer                   1

Universal PCR Primer                   1

H2O                   13

total                    50

PCR Cycling Conditions:

| Temp | Time | Cycle |
| --- | --- | --- |
| 98 | 30 sec | 1 |
| 98 | 10 sec | # of Cycles calculated Above |
| 65 | 30 sec |  |
| 72 | 30 sec |  |
| 72 | 5 min | 1 |
| 4 | Hold |  |

Zymo Elute with 10 uL

6. Size select by Gel the same as “4”

Finally dissolve in 10uL H2O and use Kappa qPCR kit to quantify