# Western using Invitrogen System

## SDS-PAGE

- Thaw cell pellets on ice

- Prepare RIPA buffer

Add	1:100	phosphatase inhibitor set I

1:100	phosphatase inhibitor set II

1:500 	protease inhibitor set III

(I usually take 500 µl RIPA and add 5 µl phos inhib I, 5 µl phos inhib II, and 1 µl prot inhib III)

- Add 20-30 l RIPA buffer + inhibitors to each pellet (typically, for 5x106 cells, I add 25 l)

- Incubate on ice 10-30 minutes

- Freeze/Thaw (optional step)

MeOH/dry ice	3 min

37°C water bath	3 min

Repeat 3 times

- Put samples in shaker (on max) at 4°C for 10 min (optional step)

- Spin at 10,000 x g for 10 minutes at 4C

- Transfer supernatant to new tube

- Add 1 µl NuPage reducing agent and NuPage sample buffer to a final concentration of 1X (stock is 4X)

- Heat sample to 70C for 10 minutes in heat block

- Load samples and molecular weight markers onto SDS-PAGE gel

- Add running buffer to inner and outer chambers

50 ml		20X NuPage MES or MOPS SDS buffer

950 ml		Deionized water

(this buffer can be reused 2-3 times)

- Run gel at 200V for 30-50 minutes

## TRANSFER

- Make up transfer buffer

50 ml 		20X NuPage transfer buffer

100 ml 	Methanol

850 ml 	Deionized water

(this buffer can be reused 2-3 times)

- Cut 4 pieces of filter paper and 1 piece of  PVDF membrane per gel to be transferred

- Soak sponges and filter paper in transfer buffer

- Soak PVDF membrane in

Methanol		30 seconds

Water			1 minute

Transfer buffer	1 minute+

- Soak gel in transfer buffer

- Assemble the gel/membrane sandwich in the blot module in the following way

Blotting pad

Blotting pad

Blotting pad

Filter paper

PVDF membrane

Gel

Filter paper

Blotting pad

Blotting pad

Cathode Core (-)

- Make sure to add enough blotting pads to rise 0.5 cm over the rim of the cathode core

- Place the anode core (+) on top of the pads and place the entire blot module in the running chamber

- Fill the inner chamber with transfer buffer until the gel/membrane sandwich is covered

- Fill outer chamber with deionized water (~650 ml)

- Transfer at 30 V for 1½ hours

- Wash at RT in PBST on shaker for 5’, 5’, and 10’ (change the wash buffer in between)

- Block in 5% milk (2.5 g nonfat dry milk in 50 ml PBST) RT on shaker for 1 hour or at 4°C overnight.

- Wash at RT in PBST on shaker for 5’, 5’, and 10’ (change the wash buffer in between)

- Primary antibody (dilute antibody in block solution*) – Incubate O.N. at 4C on shaker or for 1 hour at RT on shaker

- Wash at RT in PBST on shaker for 5’, 5’, and 10’ (change the wash buffer in between)

- Secondary antibody (dilute antibody in block solution*) – Incubate 1 hour at RT on shaker

- Wash at RT in PBST on shaker for 5’, 5’, 5’, 5’, and 10’ (change the wash buffer in between)

## DETECTION

- ECL (Amersham – ECL Plus, cat. # RPN2132) – Mix detection solutions A and B in a ratio of 40:1 (e.g., 1 ml A + 25 l B) on a piece of saran wrap

- Drain excess wash buffer from membrane and place it protein side down onto the ECL mixture

- Incubate at RT for ~4 minutes

- Drain off excess ECL mixture and wrap blot in saran wrap

- Expose blot to film and develop

*Antibodies diluted in milk can be saved and stored at -20C for reuse.

STRIPPING A BLOT

Strip buffer:	80 ml		10% SDS

25 ml		1 M Tris HCl, pH 6.7

2.9 ml		Beta mercaptoethanol

Bring up to 400 ml with water

Heat buffer to 55°C - 60°C with slow stirring (I usually do this in a 600 ml beaker).

When the buffer gets to temperature, soak membrane (PVDF) in MeOH for ~30 sec and then in PBST for ~30 sec.

Leave blot in buffer (with slow stirring) for 25-30 minutes.

Wash in PBST

Continue with blocking, primary antibody, and secondary antibody.

PBST:

46 g		Na2HPO4

11.84 g	NaH2PO4

23.36 g	NaCl

4 mL		Tween 20

Up to 4 L with water